FEB2 power-up procedure

- FEB2 power connector configuration.

(No Milano mezzanine) configuration:

Set power connector input voltage on pins 2, 3, 4 to 10V. The same voltage is going to be used for LSB.  Install H6 remove H5 and H7.

Set power connector input voltage on pins 7, 8, 9 to be in range 6 to 48V.

Install H2, H4. Remove H1, H3.

- DC-DC converter output voltages checks.

In case of not using the regulators for LpGBT12/13 1.2V rail, change R_76 to 8.66 kOhm

(Ask Nancy in advance to get 8.66-kOhm resistor as an optional resistor). Remove R_2.

In case of using the LDO for LpGBT12/13 leave R_76 as is, install R_2, and remove R_109.

1.4V and 2.5V DC-DC outputs should be on immediately after power on.

Remove 0-Ohm resistors at the output of DC-DC converters (5 of them for each DC-DC converter).

Before you continue, you may need to execute points 3.) 4.) 5.) .

Disable all DC-DC outputs and measure 0V at each DC-DC output.

Enable   all DC-DC outputs and measure nominal voltages at each DC-DC output.

- Control LpGBT external configuration pins setting.

In case that you start with control optics ready and working:

-Make sure that the mode for LpGBT12/13 is [0011] (5.12 GHz, Transceiver).

- Control LpGBT internal register configuration.

A this moment it is enough to send  40MHz clocks to all data LpGBTs. Activate all connected ECLK outputs on LpGBT12 and LpGBT13.

- 40 MHz clock checks on all data LpGBT inputs

- LDO output voltages check.

Install all 0-Ohm resistors at the output of DC-DC converters (5 of them for each DC-DC converter).

Enable   all DC-DC outputs and measure nominal voltages at each DC-DC output.

Recheck all DC-DC outputs.

Check all LDO outputs.